GEARS

- What types of gears are used for parallel, intersecting and skew shafts? Explain.

- Define the terms: Pitch circle, Pitch diameter, Pitch point, Circular pitch, Module, Pressure angle, Addendum, dedendum.

- State and prove the law of gearing. Show that involute profile satisfies the conditions for correct gearing.

- Derive an expression for the velocity of sliding between a pair of involute teeth. State the advantages of involute profile as a gear tooth profile.

- Derive an expression for the length of path of contact and length of arc of contact in a pair of spur gears in mesh. Derive an expression for contact ratio.

- What do you understand by the term ‘interference’ as applied to gears.

- Derive an expression for the minimum number of teeth required on the pinion in order to avoid interference in involute gear teeth.

- Derive an expression for the minimum number of teeth required on the pinion in order to avoid interference when it gears with a rack.

- What is undercut? Explain.

- Two gears in mesh have a module of 10mm and a pressure angle of 250. The pinion has 20 teeth and the gear has 52 teeth. The addendum of both the gears is equal to one module. Determine : i) the number of pair of teeth in contact, ii) angles of action of the pinion and the wheel,	iii) Maximum velocity of sliding

- Two mating gears have 20 and 40 involute teeth of module 10 mm and 200 pressure angle. If the addendum on each wheel is such that the path of contact is maximum and interference is just avoided, find the addendum for each gear wheel, path of contact, arc of contact and contact ratio.

- Two gear wheels mesh externally and are to give a velocity ratio of 3 to 1. The teeth are of involute form; module = 6 mm, addendum =  one module, pressure angle = 200. The pinion rotates at 90 r.p.m. Determine: 1. The number of teeth on the pinion to avoid interference on it and the corresponding number of teeth on the wheel, 2. The length of path and arc of contact, 3. The number of pairs of teeth in contact, and 4. The maximum velocity of sliding.

- A pinion with 24 involute teeth of 150 mm of pitch circle diameter drives a rack. The addendum of the rack and pinion is 6 mm. Find the least pressure angle which can be used if undercutting of the teeth is to be avoided. Using this pressure angle, find the length of arc of contact and the minimum number of teeth in contact at one time.